INTERN element_t         build_el_kreis(mx, my, ra, as, ae, attr_str)

double       mx, my, ra, as, ae;

char *       attr_str;

{

element_t    el;

double       ao;

/* winkel sind in mathematischen graden !! */

as = abs_rad(as);

ae = abs_rad(ae);

ao = angle_diff(as, ae, FSIGN(ra));

el.type = EL_ARC;

ra = fabs(ra);

el.u.prim.u.arc.tm[0][0] = ra*cos(as);

el.u.prim.u.arc.tm[0][1] = ra*sin(as);

/* des bloede fsign() ist positiv, wenn der wert negatiuv ist

und umgekeht !!  */

el.u.prim.u.arc.tm[1][0] = fsign(ao)*ra*-sin(as);

el.u.prim.u.arc.tm[1][1] = fsign(ao)*ra*cos(as);

el.u.prim.u.arc.tm[2][0] = mx;

el.u.prim.u.arc.tm[2][1] = my;

el.u.prim.u.arc.a = fabs(ao);

memcpy(el.u.prim.attr_str, attr_str, ATTR_STR_L);

el.u.prim.attr_str[ATTR_STR_L] = '\0';

return(el);

}

NAME

arc_pars() -- arc parameters

DESCRIPTION

Gets the parameters of an ellipse arc specified by a transformation matrix.

RETURN VALUE

Returns the parameters for the following ellipse equation:

x = a*cos(th)*cos(ph) - b*sin(th)*sin(ph) + pc.x

y = a*sin(th)*cos(ph) + b*cos(th)*sin(ph) + pc.y

whith

ao > 0:      as <= ph <= as+ao

ao < 0:   as+ao <= ph <= as

pc    center point

a, b  axes

th    rotation angle    (-M_PI_2 <= th <= M_PI)

ao    opening angle     ellipse arc : -M_2PI < oa < M_2PI

full ellipse: oa == M_2PI

as    starting angle of ph for an ellipse arc (-M_PI <= sa <= M_PI) (not specified for full ellipses)

ps    start point of an arc (not specified for full ellipses)

pe    end point of an arc (not specified for full ellipses)

IMPLEMENTATION NOTES

wanted ellipse:

x = a*cos(th)*cos(ph) - b*sin(th)*sin(ph) + pc.x

y = a*sin(th)*cos(ph) + b*cos(th)*sin(ph) + pc.y

with ph = as    =>  x,y = ps

ph = as+ao =>  x,y = pe

x, y are specified by the transformation matrix:

[tm'] = 1/[tm]

[x, y] = [x' y'] * tm'             x', y' are the coordinates of the untransformed unit circle

pc can be taken directly from the transformation matrix

the other parameters we get by substituting x', y' in the unitcircle through x, y

x'*x' + y'*y' = 1

A*x*x + 2*B*x*y + C*y*y = F           general equation of cone sections

A =  tm[1][1]*tm[1][1] + tm[0][1]*tm[0][1]

B = -tm[1][1]*tm[1][0] - tm[0][1]*tm[0][0]

C =  tm[0][0]*tm[0][0] + tm[1][0]*tm[1][0]

F = (tm[0][0]*tm[1][1] - tm[0][1]*tm[1][0])*(tm[0][0]*tm[1][1] - tm[0][1]*tm[1][0])

x + y = cos(th)*(a*cos(ph)+ b*sin(ph)) + sin(th)*(a*cos(ph) - b*sin(ph))

y = sin(th)*(a*cos(ph) - b*sin(ph))

x = cos(th)*(a*cos(ph) + b*sin(ph))

x*sin(th) = a*cos(ph) + b*sin(ph)

y*cos(th) = a*cos(ph) - b*sin(ph)

x*sin(th) = (a*cos(ph) + b*sin(ph))  * (a*cos(ph) + b*sin(ph))

y*cos(th) = (a*cos(ph) - b*sin(ph))  * (a*cos(ph) + b*sin(ph))

x*sin(th) = a(ph) + 2*a*b*cos(ph)*sin(ph) + b(ph)

y*cos(th) = a(ph) - b(ph)

x = a*cos(th)*((1-tan*(phi/2)) - b*sin(th)*2*tan(phi/2)/(1+tan)/(1+tan*(phi/2))

x*(1+tan(phi/2)) - b*sin(th)*2*tan(phi/2)

y*(1+tan(phi/2)) + b*cos(th)*2*tan(phi/2)

(x+y)*(1+tan(phi/2)) + (cos(th)-sin(th))*b*2*tan(phi/2)

(x+y)*(1+tan(phi/2))+b*2*tan(phi/2)) + sin(th)*(a*(1-tan(phi/2)) + b*cos(th)*2*tan(phi/2)) * (a*sin(th)*(1-tan*cos(phi/2))*sin(phi/2)

y = a(th)*(1-tan + b(th)*4*tan(phi/2)) * b*cos(th)*2*tan(phi/2))

-----

x = a(th)*(1-tan - b(th)*4*tan*cos(phi/2))*sin(phi/2) + 2*(a*sin(th)*(1-tan*cos*cos(phi/2)*sin(phi/2)

x = a*cos(th)*cos(ph) - b*sin(th)*sin(ph) + pc.x

y = a*sin(th)*cos(ph) + b*cos(th)*sin(ph) + pc.y

x    cos(ph)      a*cos(th)   a*sin(th)

y =  sin(ph) *  - b*sin(th)   b*cos(th)

0           0

*/

void                 arc_pars(arc, pc, a, b, th, as, ao, ps, pe)

world_arc_t          arc;

world_coord_t *      pc;

world_t *            a;

world_t *            b;

abs_angle_t *        th;

abs_angle_t *        as;

rel_angle_t *        ao;

world_coord_t *      ps;

world_coord_t *      pe;

{

double               A, A_, B, C, F;

world_coord_t        p;

pc->x = arc.tm[2][0];

pc->y = arc.tm[2][1];

A = arc.tm[1][1]*arc.tm[1][1] + arc.tm[0][1]*arc.tm[0][1];

B = -arc.tm[1][1]*arc.tm[1][0] - arc.tm[0][1]*arc.tm[0][0];

C = arc.tm[0][0]*arc.tm[0][0] + arc.tm[1][0]*arc.tm[1][0];

F = (arc.tm[0][0]*arc.tm[1][1] - arc.tm[0][1]*arc.tm[1][0])*(arc.tm[0][0]*arc.tm[1][1] - arc.tm[0][1]*arc.tm[1][0]);

if (A-C != 0.0)

{

*th = atan((2.0*B)/(A-C))/2.0;

if (C < A)

A_ = (A+C)/2.0 + sqrt((A-C)*(A-C)+4.0*B*B)/2.0;

else

A_ = (A+C)/2.0 - sqrt((A-C)*(A-C)+4.0*B*B)/2.0;

*a = sqrt(F/A_);

*b = sqrt(F/(A+C-A_));

}

else

{

*th = 0.0;

*a = sqrt(F/A);

*b = sqrt(F/C);

}

if (arc.a == M_2PI)

{

*as = 0.0;

*ao = M_2PI;

}

else

{

if (arc.a < 0.0)

{

ps->x = cos(arc.a);

ps->y = sin(fabs(arc.a));

pe->x = 1.0;

pe->y = 0.0;

}

else

{

ps->x = 1.0;

ps->y = 0.0;

pe->x = cos(arc.a);

pe->y = sin(fabs(arc.a));

}

p.x = arc.tm[0][0]*ps->x + arc.tm[1][0]*ps->y;

p.y = arc.tm[0][1]*ps->x + arc.tm[1][1]*ps->y;

*ps = p;

p.x = arc.tm[0][0]*pe->x + arc.tm[1][0]*pe->y;

p.y = arc.tm[0][1]*pe->x + arc.tm[1][1]*pe->y;

*pe = p;

*as = atan2(ps->y, ps->x);

*ao = angle_diff(*as, atan2(pe->y, pe->x), arc.a < 0.0 ^ tm_reflection_p(arc.tm) ? -1 : 1);

*as = ABS_RAD(*as-*th);

ps->x += pc->x;

ps->y += pc->y;

pe->x += pc->x;

pe->y += pc->y;

}

}

NAME

angle_diff() -- angle difference

DESCRIPTION

Calculates the difference between two angles given in radians. If direction == 1 the difference is calculated clockwise,

else (direction == -1) counterclockwise.

RETURN VALUE

Difference between to angles in radians. The sign depends on direction.

*/

double      angle_diff(sa, ea, direction)

double      sa, ea;      /* start angle, end angle */

int         direction;

{

if (direction == 1)

if (sa <= ea)

return(ea-sa);

else

return(ea-sa+M_2PI);

else

if (sa >= ea)

return(ea-sa);

else

return(ea-sa-M_2PI);

}

/**/

NAME

tm_reflection_p() -- transformation matrix reflection prove

DESCRIPTION

Proves whether an homogeneous transformation matrix contains a reflection by one and only one axis.

RETURN VALUE

FALSE   no refletion

TRUE    reflection

IMPLEMENTATION NOTES

[ r*cos(th)   r*sin(th)]

[x, y] * [-s*sin(ph)   s*cos(ph)] = [r*cos(th)*x - s*sin(ph)*y + xt, r*sin(th)*x + s*cos(ph)*y + yt]

[    xt          yt    ]

If we assume that th and ph are in the same quadrant a reflection can only happen if the signs of r and s are

different.

*/

boolean             tm_reflection_p(tm)

trans_matrix_t      tm;

{

if (fabs(tm[0][0]+tm[1][1]) > fabs(tm[0][1]+tm[1][0]))   /* use the greater values for comparison */

return (FSIGN(tm[0][0]) != FSIGN(tm[1][1]));

else

return (FSIGN(tm[0][1]) == FSIGN(tm[1][0]));

}